Supplier Meeting Summary

Modular All Drives — EOL Test System | Develop LLC (USA)

| Supplier | Develop LLC — ~2.5 h north of Chicago, Wisconsin, USA |
| --- | --- |
| Date / Duration | First discovery call — ~30 min |
| Meeting type | Introductory call — concept walkthrough & capability discussion |
| Develop attendees | Matt Moseman (Sales) |
| WEG attendees | Aluizio Kirschner & Raphael Franklim (Industrial Engineering); Júlio Cesar Pedroso; Alissonn Felippe Bebber (assembly line) |

# Executive Summary

Develop LLC is a US machine builder (Wisconsin, north of Chicago) that Matt repeatedly described as a strong technical fit — "right in our wheelhouse" — for the EOL test concept. They engaged well technically (connection strategies, fire suppression, modular staggering of load modules) and the end-2027 timeline aligns with their booking (projects starting Q1 2027). The decisive issue is geographic: as a matter of company policy Develop serves only North America (the lower 48 US states) and will not commission or support in Brazil.

Overall fit: Technically strong, but a structural blocker on Brazil support. They build the machine in the US and require US-based site acceptance; on-site commissioning and after-sales in Brazil are outside their model. The only path offered is Develop building and managing a dedicated team in Brazil at additional cost passed on to WEG.

# Company Profile (as presented)

- US machine builder located ~2.5 hours north of Chicago (Wisconsin); works in metric.

- Serves North America only — the lower 48 US states; this is a stated company policy, not a capability limit.

- Security-conscious sales process: requires a live human call before opening quote PDFs, due to phishing risk (their scope email was caught by filtering).

- Experience includes assembly lines (mostly under NDA) — cited lighting industry: PCB screwing/gluing, adhesive dispensing, high-precision plastic + electronics assembly.

- Operates a model for building and managing dedicated regional teams for customers who can't find local support — already doing this for other customers.

# Capability Assessment Against Scope

| Requirement | Assessment | Notes |
| --- | --- | --- |
| EOL electrical test (hipot / functional / load) | Strong fit | Matt called it perfectly aligned and "right in our wheelhouse"; no concerns about building it. VFD size/power confirmed as nothing extreme (220–575 V, ~60 A, ~50×30 cm, ~30 kg). |
| Automated electrical connection (product→fixture) | Capable | Has used large pogo pins and pneumatic clamping to engage contact points. Offered to help scope design-for-automation of the product, or evaluate manual connection — depends on accessibility. |
| Conveyor / flow-through / modular expansion | Strong | Proposed a simpler motion: drop the shuttle slightly and pneumatically lift it rather than pushing it back; both approaches keep the conveyor free. Understood staggering/stacking of load modules behind shared hipot + functional. |
| Safety / fire suppression | Proactive | Raised fire suppression unprompted; recommended an oxygen-displacing canister system (as used in titanium machining) inside the cabinet — a useful addition to WEG's scope. |
| Modular, separately-orderable modules | Aligned (after discussion) | Initially preferred one integrated machine (cheaper); accepted WEG's need for separate modules so capacity can grow by adding individual load/functional modules. Understood once cycle-time imbalance was explained. |
| Brazil commissioning & after-sales support | Blocker | Company policy: serves North America / lower-48 only. Will build the machine but requires US-based site acceptance; will not commission or support in Brazil and has no Brazil partner. |
| Dedicated Brazil team (alternative) | Possible — at cost | Open to setting up and managing a dedicated team in Brazil (hire locally, second a US team member to run it). Machines still built in US; all costs passed on to WEG. Driven by their MTTR/downtime philosophy. |
| Assembly line incl. PCB handling | Some experience | Has done PCB screw/glue/dispense and plastic+electronics assembly (lighting). References mostly under NDA. Assembly handled as a separate project/quote (Alissonn/Franklim to send scope). |

# Key Technical Points Aligned

- VFD specs confirmed: 220–575 V, ~60 A; largest ~50×30 cm, ~30 kg — Matt considered this well within scope.

- Fixture→module connection standardized; product→fixture connection product-specific; sub-plate per module in the fixture.

- Cycle times confirmed: hipot 3–60 s; functional 1.5–7 min; load 6–20 min — load is the bottleneck; ~1 hipot + 1 functional feed 3–5 load modules.

- Only the load module is heated. Develop suggested cabinet fire suppression for failures.

# Commercial & Timeline

| WEG project deadline | Test system commissioned in Brazil by end of 2027 |
| --- | --- |
| Develop timeline | Booking projects for Q1 2027 start; sees no problem building by end of 2027 even starting January 1 |
| Quote purpose | Budgetary target; separate quotes for testing line vs assembly + packaging line |
| Budget position | No fixed budget — evaluated on payback (as communicated to all suppliers) |
| Selection timing | Procurement decision targeted by end of 2026; selection over the coming months (July/Aug) |
| Replication | Concept to be replicated to China, Italy, Mexico, possibly USA |

# Action Items

| Owner | Action | Status |
| --- | --- | --- |
| WEG | Resend the scope / technical specification document to Matt (caught by email filtering); send the concept visuals. | Open |
| WEG (Assembly) | Alissonn to send the assembly-line scope; continue by email. | Open |
| Develop | Share with engineering and prepare a proposal once documentation is received. | Open |
| WEG | Decide whether the dedicated-Brazil-team model is worth exploring given the no-Brazil-support policy. | Open |

# Evaluation Notes

Positives

- Strong technical alignment and genuine enthusiasm; engaged on connection methods, fire suppression and modular staggering.

- Practical, lower-cost mechanical ideas (drop-and-lift shuttle) and proactive safety thinking.

- Timeline fits comfortably; security-conscious and professional.

Watch points

- Hard blocker: no Brazil commissioning or support (North-America-only policy). The dedicated-team workaround adds cost and complexity.

- US build + US site acceptance means WEG would own commissioning in Brazil unless the paid team model is used.

- Assembly references largely under NDA — capability harder to verify.

- WEG-component preference (PLCs, sensors, VFDs) noted; Matt confirmed they already use WEG inverters in cabinets, but component strategy not yet detailed.

Prepared from the meeting transcript. Standard qualification questions applied for cross-supplier comparison.