Supplier Meeting Summary

Modular All Drives — EOL Test System | MASMEC (Italy)

| Supplier | MASMEC S.p.A. — Bari (southern Italy); all machines built in Italy |
| --- | --- |
| Date / Duration | 16 June 2026, 11:00 (BRT) — 1 h 14 min |
| Meeting type | First discovery call — company & inverter-test capability presentation, concept review |
| MASMEC attendees | Stefano Piccinno (Sales Engineer); Luigi Di Leo |
| WEG attendees | Aluizio Kirschner & Raphael Franklim (Industrial Engineering); Júlio Cesar Pedroso; Alissonn Felippe Bebber (assembly line) |

# Executive Summary

MASMEC presented as a focused Italian automation specialist (~300 employees, ~€50M target revenue, ~84% automation, 44,000 m² plant in Bari) building assembly and test systems primarily for the automotive sector, plus a Biomed division. Crucially, they showed direct, productionised experience testing automotive inverters — hipot, functional, and load test — with automatic connection of DC/AC power, comms, low-voltage supply and coolant, on pallet-based conveyor lines very close to WEG's concept. This is the most directly relevant inverter-test reference among the suppliers.

Overall fit: Strong and well-matched on inverter electrical test. They run a per-job dedicated project team (PM, sales, project engineer, buyer, assembly leader) and an R&D department for test development. Open points: LabVIEW default (WEG wants C#/Java/Python), Siemens-PLC preference vs WEG's mandated PLC, no inverter PCB-board assembly experience, and a hybrid Brazil support model.

# Company Profile (as presented)

- HQ and sole manufacturing site in Bari, southern Italy (44,000 m²); presence in Europe, China, North and South America (some installations).

- ~300 employees; ~€50M target revenue, ~84% from automation. Divisions: Automation (assembly + test) and Biomed (lab automation/healthcare).

- Per-order dedicated 5-person team: project manager, sales engineer, project engineer, buyer, assembly leader — for full control of job execution.

- Technologies: vision, laser marking/guidance, AI; digital twins via plant simulation, virtual & augmented reality, IoT machine-performance monitoring.

- Largest single order to date >€15M. References across e-motor/axle assembly, battery packs, combustion engine, transmission lines.

- All machines produced in Italy (confirmed) — relevant given WEG's concern about suppliers who represent in Europe but build in China.

# Capability Assessment Against Scope

| Requirement | Assessment | Notes |
| --- | --- | --- |
| Inverter EOL electrical test (hipot / functional / load) | Strong | Showed productionised inverter test lines: hipot (insulation resistance, leakage current), continuity/equipotential, functional (idle current, HV-DC consumption, passive/active discharge, resolver emulation, sensor/temperature calibration), and load test. Functional sequence ~12 min, parallelised across benches to hit <1 min takt. |
| Load test — passive vs active / electronic AC load | Strong (active to confirm) | Confirmed passive-load experience (inductive reactor) and brushless-motor dynamometer benches. Active / electronic AC (regenerative) load to be cross-checked with R&D. WEG currently uses AC-motor dynamometers but is moving toward electronic AC load — left open for MASMEC to propose. |
| Automatic connection (DC/AC power, comms, LV, coolant) | Strong | Demonstrated automatic DC/AC, comms and LV connection plus coolant fill/drain to extreme temperature. Builds or sources contacting (bus bars/needles) — partners with specialists (e.g. Creatic) when no proven in-house contact exists. |
| Automated product→fixture connection | Capable — conditional | Possible only if the product connectors are designed for automated handling (defined gripping/locating surfaces). If not design-for-automation, manual is more realistic. WEG accepts manual fallback if stated in the quote. |
| Conveyor / flow-through / modular expansion | Strong | Proposed Bosch Rexroth conveyor (likely TS2) whose catalogue already supports the flow-through / main-line-free function WEG described. Understood backward-move-to-test-position concept. |
| Heated load module (50 °C) | Strong | Can implement a climatic chamber but noted it may be over-specified; WEG confirmed fixed 50 °C, temperature only, no humidity — so a simpler, more cost-effective heating solution applies. |
| Brazil commissioning & after-sales support | Capable — hybrid | Has installed in Mexico and Brazil before. MASMEC team travels for installation through final acceptance; may partner with a local company for routine support, specialists from Italy for complex issues. Warranty terms apply. |
| Use of WEG components / PLC | Partial risk | Builds with Siemens PLC and would prefer it (automation level on the bench is low). WEG pushed back: as a PLC/VFD maker, using Siemens is not acceptable; WEG will supply drives, PLCs and safety hardware. MASMEC to check internally. |
| Software language / ownership | Risk | Default LabVIEW + National Instruments HW. Confirmed C# (and other languages) possible — to cross-check with software dept; non-NI hardware also possible if not using NI software. WEG requires open-source C#/Java/Python. |
| Inverter PCB-board assembly | Not in portfolio | No experience assembling the electronic board itself inside the drive; has done board-level/part handling and is ESD-capable, but inverter board build is not yet in their portfolio (open to it). Relevant to the separate assembly-line scope. |

# Key Technical Points Aligned

- Modular, scalable/repeatable concept understood — module treated as a product; capacity grows by adding modules.

- Drive moves backward to a test position so the conveyor stays free for parallel testing; Bosch Rexroth conveyor supports this natively.

- Fixture→module connection standardized; product→fixture connection product-specific.

- Cycle times confirmed: hipot 3–60 s; functional 1.5–7 min; load 6–20 min — load is the bottleneck; ~1 hipot + 1 functional feed 3–5 load modules.

- Functional-test detail (IOs, measurements, motor tests) to be defined jointly later — WEG holds the test know-how.

- MASMEC offered to build a 3D concept of the line during the proposal phase if time allows.

# Commercial & Timeline

| WEG project deadline | Test + assembly lines commissioned in Brazil by end of 2027 (MASMEC sees this as a workable window) |
| --- | --- |
| Quote purpose | First quote is a budgetary target only — no detailed layouts/datasheets required yet |
| Quotes requested | Three separate quotes: (1) testing line, (2) assembly line, (3) full line together |
| Quote scope (this stage) | Supplier to quote all components incl. WEG-type material; WEG-component discount discussed later |
| Budget position | No fixed budget — evaluated on payback (as communicated to all suppliers) |
| Replication | Concept to be replicated to plants in China, Italy, Mexico |

# Action Items

| Owner | Action | Status |
| --- | --- | --- |
| MASMEC | R&D to cross-check active / electronic AC (regenerative) load experience and revert. | Open |
| MASMEC | Confirm with software dept the C#/Java/Python (non-LabVIEW) and non-NI hardware options. | Open |
| MASMEC | Check internally on using WEG-mandated PLC instead of Siemens. | Open |
| MASMEC | Bring R&D/inverter specialist into the loop for the functional/load test details. | Open |
| MASMEC | Prepare three budgetary quotes (testing / assembly / full line) as soon as possible. | Open |
| WEG | Provide test specification and product details; support software development collaboratively. | Open |
| WEG (Assembly) | Alissonn & Franklim to send the assembly-line scope by email (separate project). | Open |

# Evaluation Notes

Positives

- Most directly relevant inverter-test reference: hipot, functional and load already productionised, with automatic power/comms/coolant connection on conveyor lines.

- Engaged critically (design-for-automation of connectors, humidity vs temperature, takt parallelisation) and was honest about what needs R&D cross-checking.

- All-Italy manufacturing and a dedicated per-job team; prior installs in Brazil and Mexico.

Watch points

- Siemens-PLC preference conflicts with WEG's requirement to use its own PLC — a clear point of negotiation.

- LabVIEW/NI default vs WEG's open-source C#/Java/Python requirement.

- No inverter PCB-board assembly experience — limits their fit for the full assembly-line scope.

- Brazil support is hybrid (local partner + Italy specialists) — clarify response times and warranty boundaries.

Prepared from the meeting transcript of 16 June 2026. Standard qualification questions applied for cross-supplier comparison.