Supplier Meeting Summary

Modular All Drives — EOL Test System | PIA Automation (China)

| Supplier | PIA Automation — China |
| --- | --- |
| Date / Duration | 18 June 2026, 08:02 (BRT) — 26 min |
| Meeting type | First discovery call — concept walkthrough & technical Q&A (no PIA presentation this round) |
| PIA attendees | Abel Xiang Wang; David Qingjun Cui |
| WEG attendees | Aluizio Kirschner, Raphael Franklim, Carlos Roberto de Andrade (Industrial Engineering); Júlio Cesar Pedroso |

# Executive Summary

A short, focused introductory call. PIA did not present (they offered a fuller technical presentation — test approach, electrical/mechanical design — for a follow-up). They confirmed automated assembly-line / robotics experience and engaged with practical, well-targeted test questions (hipot pin grouping, 4 kV selection, HMI vs PLC control, RS-485/USB testing, load-temperature control). Importantly, PIA confirmed they can commission and support in Brazil and already have a Brazil project (an automatic connector-assembly machine).

Overall fit: Promising — engaged technically and is one of the few overseas suppliers confirming Brazil commissioning + production support plus an existing local reference. Depth of inverter-specific test/assembly experience is not yet demonstrated (deferred to a follow-up). Quote expected next week (after a 3-day holiday in China).

# Company Profile (as presented)

- PIA Automation (China) — automation supplier; confirmed substantial experience with automated assembly lines and robotics.

- No company presentation given this round; offered to bring a detailed technical presentation (how to test, electrical/mechanical design selection) to a follow-up meeting.

- Has an existing project in Brazil — a fully automated connector-assembly machine (location near a capital, to be confirmed).

- PCB assembly: confirmed yes; will share a similar-process example by email (none available live).

# Capability Assessment Against Scope

| Requirement | Assessment | Notes |
| --- | --- | --- |
| EOL electrical test (hipot / functional / load) | Engaged — to demonstrate | Asked sharp, relevant questions but did not yet show inverter-test references. Understood hipot scope (power connections only, 4 kV max), functional (control connections), and load (heated). Offered a deeper technical presentation next time. |
| Hipot details | Understood | Probed pin grouping / how many contact groups and the mechanism to raise pins to the product; confirmed 4 kV max and that hipot needs only power connections (no control). |
| Functional test — control architecture | Aligned (after clarification) | Initially assumed an HMI/vision-monitored approach; WEG (Júlio) clarified the PLC controls the product directly via RS-485 / Modbus — no HMI test needed. PIA accepted. |
| RS-485 / USB testing | Capable — to define | RS-485 fine. For USB, WEG noted a host (PC) is needed; whether USB testing is required to be decided later in detail. |
| Automated electrical connection (product→fixture) | Capable | Engaged on the pin-raising mechanism and connection groups; understood the fixture-with-standard-interface concept. Requested the product power/control connector images (WEG to send). |
| Conveyor / flow-through / modular expansion | Routine | Called the conveyor solution "very common"; understood the free-conveyor / backward-move-to-test-position concept for parallel testing. |
| Heated load module (50 °C) | OK | Confirmed 50 °C is the fixed maximum; will select temperature-control hardware accordingly. |
| Use of WEG components / WEG PLC | Flexible | WEG PLC suggested but not mandatory; PIA would normally use the customer's preferred PLC — to state clearly in the quote. |
| Brazil commissioning & support | Confirmed | Can do site commissioning and production support in Brazil; already has a Brazil project (automated connector assembly). |
| Inverter PCB / product assembly | Yes — to evidence | Confirmed PCB-into-product assembly experience; will send a similar-process example by email. Handled as the separate assembly-line scope (Franklim/Alissonn). |

# Key Technical Points Aligned

- Modular, globally-replicable concept (Brazil first, then China, Italy, Mexico); capacity grows by adding modules, each with its own conveyor.

- Drive moves backward to a test position so the conveyor stays free for parallel testing; fixture→module connection standardized, product→fixture product-specific.

- Hipot: power connections only, 4 kV max. Functional: PLC controls the product directly via RS-485/Modbus (no HMI test).

- Cycle times confirmed: hipot 3–60 s; functional 1.5–7 min; load 6–20 min — load is the bottleneck; ~1 hipot + 1 functional feed 3–5 load modules.

- Only the load module is heated (50 °C fixed max).

- This stage is budgetary only — target price, no deep detailing; test sequence to be shared collaboratively if the project advances.

# Commercial & Timeline

| WEG project deadline | Test + assembly lines commissioned in Brazil by end of 2027 |
| --- | --- |
| Quote purpose | Budgetary target price only — no detailed engineering at this stage |
| Quotes requested | Three quotes: (1) testing line, (2) assembly line, (3) full line together |
| PIA quote delivery | Will start generating the quote and aim to deliver next week (3-day holiday in China first) |
| Budget position | No fixed budget — evaluated on payback (as communicated to all suppliers) |
| Replication | Concept to be replicated to China, Italy, Mexico |

# Action Items

| Owner | Action | Status |
| --- | --- | --- |
| WEG | Send the product power/control connector images and the test specification to PIA for quoting. | Open |
| WEG (Assembly) | Alissonn/Franklim to send the assembly-line scope (drive details, sizes) by email. | Open |
| PIA | Send a similar-process PCB/assembly reference example by email. | Open |
| PIA | Prepare and deliver three budgetary quotes (testing / assembly / full line) — targeting next week. | Open |
| PIA | Prepare a detailed technical presentation (test approach, electrical/mechanical design) for a follow-up meeting. | Open |

# Evaluation Notes

Positives

- Confirmed Brazil commissioning + production support and an existing Brazil reference — a strong answer to the recurring support gap, rare among overseas suppliers.

- Asked focused, knowledgeable test questions (pin grouping, control architecture, RS-485/USB, temperature control).

- Flexible on PLC/components; confirmed assembly-line and PCB experience.

Watch points

- No references shown yet — inverter-specific test/assembly capability still to be evidenced (follow-up presentation pending).

- Short call; commercial/support claims (Brazil project location, support model) need confirming in writing.

- Software language preference (C#/Java/Python) not discussed — to raise.

- Build location is China — confirm logistics and any sourcing/compliance considerations vs the Brazil-installed timeline.

Prepared from the meeting transcript of 18 June 2026. Standard qualification questions applied for cross-supplier comparison.