# Test Specification

Supplier Reference | Rev. 00 - June 2026 |

## Test Sequence

## Visual Inspection

Verify HMI display is not damaged.

## Applied Voltage Test (Hipot)

Short-circuit all power terminals: input, DC bus, output.

Apply DC voltage between shorted terminals and chassis ground:

220 V models (T2): 2100 Vdc / 2 s

380-480 V models (T4): 2800 Vdc / 2 s

500-600 V models (T5): 3111 Vdc / 2 s

Pass: no insulation breakdown, leakage current max 4 mA.

Voltage must ramp up and down over 1 s at start and end.

## Control Power Tests

External 24 V supply: power inverter via 24 V +/-10% -- HMI must start without faults.

Internal 24 V output: apply 250 mA load -- verify 21.6 to 26.4 V.

## Firmware and Model Programming

Load firmware and model configuration via Ethernet or RS-485.

Power-cycle the inverter -- must restart without errors.

Confirm model identifier matches the unit under test.

## Serial Number and MAC Address

Write serial number and Ethernet MAC address via Modbus during the test routine.

## Communication Interfaces

Ethernet Port A: Modbus TCP read must succeed.

Ethernet Port B: same test repeated.

RS-485: Modbus RTU -- Read-Device-ID must return correct identification.

## Analog Inputs

Current mode 0 to 20 mA: verify reading at 0 / 4 / 10 / 16 / 20 mA -- tolerance +/-2%.

Voltage mode 0 to 10 V: verify reading at 0 / 2 / 5 / 8 / 10 V -- tolerance +/-2%.

## Analog Outputs

Voltage mode 0 to 10 V: verify output at 0 / 20 / 50 / 80 / 100% -- tolerance +/-0.2 V.

Current mode 0 to 20 mA: verify output at 0 / 20 / 50 / 80 / 100% -- tolerance +/-0.4 mA.

## Digital Inputs (DI1 to DI6)

Test all 6 inputs in source (PNP) and sink (NPN) configurations.

Pass: parameter status correctly reflects each input state.

## Digital Output (DO1)

OFF state: load voltage must be 0 V +/-0.2 V.

ON state: load voltage must be 21.6 to 26.4 V.

## Control Supply Voltages

24 V IO supply: 21.6 to 26.4 V (20.0 to 26.4 V when using external supply).

RTC battery: 2.85 to 3.15 V.

## RTC Test

Set current date and time via parameter.

Verify after 5 s and after a power cycle -- tolerance +/-1 s.

## Fan Speed

Enable fans via parameter and verify speed for each mechanical size -- tolerance +/-10%.

## Motor Operation

Connect a motor or a electronic load of the same rated power as the inverter under test.

Run across full speed range: 3 to 60 Hz.

Pass: current reading within +/-5%.

## STO - Safe Torque Off

OSSD mode: with STO signals OFF, inverter must enter STO state without faults.

Turn ON STO signals -- inverter goes to Ready, motor runs normally.

Trip STO while motor is running -- motor must stop with no fault generated.

Dry contact mode: repeat the same sequence using relay contacts driven by SP1/SP2 test pulses.

## Dynamic Braking (DB models only)

Trigger braking circuit -- verify if there is voltage under the braking connectors.

## Protection Tests

Ground fault: short phase W to the ground with a contactos just a pulse – fault must be indicated.

Short circuit: short phases U and V with a contactor just a pulse-- fault must be indicated.

After each test: remove fault, release inverter, verify normal operation resumes.

## Load Test

Run at 55 Hz, ambient temperature 50 °C, apply current derating per user manual.

Pass: continuous operation for at least 20 minutes without any fault.

## End of Test

Load factory defaults.

De-energize the inverter.